Self-Balancing Robot (Inverted Pendulum on Wheels)

IDEA

How it works

- An IMU sensor (accelerometer + gyroscope) measures the robot’s tilt angle

- A Kalman filter (or complementary filter) fuses the sensor data for a clean angle estimate

- A PID controller computes motor corrections in real time to keep it balanced

- Two DC motors with encoders drive the wheels accordingly

- (Optional) Full LQR state-space controller

Suggested Stack

| Component | Suggested Part | Approximate Cost |
| --- | --- | --- |
| Microcontroller | Arduino | R80 – R150 |
| IMU | MPU-6050 (6-axis) | R60 - R100 |
| Motor Driver | L298N Dual H-Bridge | R60 - R90 |
| DC Gear Motors (x2) | With encoders, 6V-12V | R200 – R400 |
| Wheels (x2) | 65 mm rubber wheels | R60 – R120 |
| Chassis | Laser-cut acrylic or 3D printed | R150 – R300 |
| Battery | 2S LiPo or 4x AA pack | R100 – R200 |
| Misc. (wiring, standoffs, switches) | --- | R100 – R150 |
| Total |  | ~R810 – R1510 |

Leaving room to add Raspberry Pi for data logging, a Bluetooth module for tuning PID gains wirelessly, or even an LCD to display live sensor values.

Where to buy

- Micro Robotics (Stellenbosch)

- Communica (Paarden Eiland)

- RS Components SA (Online)

- DigiKey South Africa (Online)

Milestones

- Get the IMU reading – wire up the MPU-6050, read raw data, implement a complementary filter to get a clean tilt angle.

- Drive the motors – test the H-bridge, get basic forward/back working

- Close the loop – implement a basic P controller first, observe instability, then add I and D terms

- Tune the PID – this is where the fun (and frustration!) is

- Stretch goal – implement an LQR controller and compare performance to PID

External Links

- https://www.instructables.com/Self-Balancing-Upside-Down-Pendulum/

- https://www.shaysackett.com/inverted-pendulum-robot/

- https://www.diva-portal.org/smash/get/diva2:916184/FULLTEXT01.pdf

- https://youtu.be/jP7Y_BfuGDI?si=Myiii08xSY3t1dM1 - Balancing robot tips video.

Objectives

- Get it to balance “horizontally”.

- Get it to balance “vertically”.

- Implement cascaded PID loop (velocity = 0).

- Try to implement LQR?

VERSION HISTORY

Idea is to do a prototype with version 2 and then upgrade the motors to version 3.

| Version | Description of Change |
| --- | --- |
| v1 | First draft |
| v2 | Changed to DRV8833 motor driver |

VERSION 2

COMPONENTS

MPU-6050

- https://youtu.be/rpCcZZJeodY?si=zEn9Rm8-psDM20C_ - YouTube video.

- https://forum.arduino.cc/t/getting-started-with-the-mpu6050-beginner-friendly-guide-with-visual-explanations/1364064 - Arduino forum explanation (other useful links).

- https://www.electronicwings.com/sensors-modules/mpu6050-gyroscope-accelerometer-temperature-sensor-module - Communica link.

Arduino

- Nano or Mega (probably Mega as it will make a better body “shape”). Going to use Uno (correct size and I am familiar with it).

Motor Driver

- DRV8833 (see link below).

- https://www.communica.co.za/products/hkd-ramps-stepper-driver-drv8833?variant=40351922618441&sfdr_ptcid=31591_617_692972130&sfdr_hash=f1fc32f8c8e4b23f0e60c33591a415d0&utm_source=www.communica.co.za&gad_source=1&gad_campaignid=22552818411&gbraid=0AAAAADLTnIZTVU6Cje37BAAegSC4YUWFZ&gclid=CjwKCAjw8arQBhB9EiwAfIKdQvWFYp-EkF1O7b7p8GFMhgPHTlYBra40T-tZbtKAcBej18Wki_dD6hoCPh4QAvD_BwE.

- Adafruit themselves recommend the DRV8833 motor driver for these motors, as it works well down to 3V and can handle the maximum current of the motors.

- https://youtu.be/cBS9IGJ8LQc?si=vVlXfKhfLhNzHTyi – DRB8833 tutorial.

- Motor voltage 2.7 V to 10.8 V, continuous current 1.2 A and peak current 2 A per motor.

- DRV8833 Pin explanation:

| Pin | Purpose |
| --- | --- |
| EEP | Puts device in sleep mode (0V = sleep, 5V = On) (Enabled by default due to J1) |
| Output 1 and Output 2 | Connects to Motor A |
| Output 3 and Output 4 | Connects to Motor B |
| ULT | Fault pin (Goes low if error, floats normally) |
| Input 3 and Input 4 | Set one high and one low to control Motor B direction |
| GND | Ground (must ground to Arduino ground and battery-) |
| VCC | Connect to positive terminal of battery |
| Input 1 and Input 2 | Set one high and one low to control Motor A direction (digitalWrite) |

DC Gear Motors (x2)

- What motors to use? Standard ones from Micromouse project.

Wheels

- Standard wheels we used for micromouse (see below).

- If struggling with control, larger wheels (diameter) can improve stability.

Chassis

- 3D print?

- Use breadboard?

- Wood?

- Wooden boards should be approximately 70 mm x 110 mm x 3 mm.

- Will need dowels to hold the different levels. These should be 6 mm in diameter (or less) and 50 mm to 60 mm high.

Battery

- Can power from laptop to start (only motors will need batteries).

- https://www.robotics.org.za/603450 - 7.4V battery.

- Will need a charging circuit too.

- https://www.robotics.org.za/DFR0564 - Battery charger circuit (USB).

- Do I need JST PH connector to connect to breadboard (something like below)? I think so.

- https://www.communica.co.za/products/xy132-02st - Communica version.

Potentiometers

- https://www.robotics.org.za/index.php?route=product/category&path=25_327 – general page.

- https://www.robotics.org.za/WH148-10K - cheaper option and pack of four.

Switch

- Needs to be 12 V, 1 A rated.

- Must be DC.

- Ideally fits in a breadboard.

- https://www.robotics.org.za/KBT001129-BLU

Other

- Nice small breadboard with multiple dividers? See below link.

- https://www.communica.co.za/products/bmt-breadboard-8-3x5-5cm-400tp?variant=52114451857708&sfdr_ptcid=31591_617_756176052&sfdr_hash=bb756a0b0dccbe2ed9b1a7c65af875e6&utm_source=www.communica.co.za&gad_source=1&gad_campaignid=20668254388&gbraid=0AAAAADLTnIbzvEAdrt-ixEG3Gu6-UzOf6&gclid=CjwKCAjw8arQBhB9EiwAfIKdQkSSMzc7uims97IhZc9b69lC9DkFK7bfecUCEdPY-fcZT8k1Mxqk8hoCdSUQAvD_BwE.

- Nice connectors.

SCHEMATIC / CONNECTION DIAGRAM

- Table and diagrams here to track how many pins I need / what is connected to what.

| Component | Connects To |
| --- | --- |
| MPU-6050 |  |
| VCC | Arduino 5V |
| GND | Arduino GND |
| SCL | Arduino SCL |
| SDA | Arduino SDA |
| DRV8833 |  |
| EEP | --- |
| Out 1 | Motor A |
| Out 2 | Motor A |
| Out 3 | Motor B |
| Out 4 | Motor B |
| ULT | --- |
| IN 4 | Arduino Digital Output Pin |
| IN 3 | Arduino Digital Output Pin |
| GND | Battery- and Arduino GND |
| VCC | Battery+ |
| IN2 | Arduino Digital Output Pin |
| IN1 | Arduino Digital Output Pin |

Arduino Uno has 14 digital pins (labelled 0 to 13). This should be plenty.

VOLTAGE & CURRENT ANALYSIS

| Device | Voltage |
| --- | --- |
| Battery Output | 7.4 V |
| Arduino Voltage Input | 7 V to 12 V (Vin Pin) |
| Arduino Digital Output Pins | 5 V |
| Motor Driver Input Voltage | 2.7 V to 10.8 V |
| DC Motors | 3 V to 6 V |
| Motor Driver Output | 7.4 V – 0.12 V = 7.28 V (must use PWM) |
| MPU6050 Voltage Supply | 3 V to 5 V |

** Motors will see roughly 7.28 V. Must drive DRV8833 with PWM signal so that it receives approximately 5 V. Measure with multimeter!

Current

| Device | Current |
| --- | --- |
| DC Motors @6V no-load | 160 mA |
| Driver Continuous Current Limit | 1.2 A (per motor) |
| DC Motors @6V when stalled | 1.5 A |
| Driver Peak Current Limit | 2 A (per motor) |
| MPU6050 Current Draw | 3.9 mA to 5.1 mA |
| Arduino Uno 5V Pin Current Limit | 800 mA |
| Arduino Digital I/O Pins | 20 mA (40 mA absolute max) |
| 10k Potentiometer | 0.5 mA |

** Peak motor driver current is not exceeded. MPU6050 and Potentiometers can be powered from standard Arduino Digital Output pin.

BILL OF MATERIALS

| Component | Price | Quantity | Total | Link |
| --- | --- | --- | --- | --- |
| Arduino Uno | R139 | 1 | R139 | Communica |
| Breadboard | R15 | 1 | R15 | Communica |
| MPU6050 | R60 | 1 | R60 | Communica |
| JST-Connector | R0.4 | 3 | R1.2 | Communica |
| Female Header | R3.39 | 2 | R6.78 | Communica |
| DRV8833 | R25 | 1 | R25 | Communica |
| Communica Total |  |  | R202.98 |  |
| Geared DC Motor | R48.26 | 2 | R97 | Digikey |
| Robot Wheel | R73.62 | 1 | R74 | DigiKey |
| Male Connectors | R31.90 | 1 | R31.90 | DigiKey |
| Button | R4.91 | 5 | R24.55 | DigiKey |
| Vero Board | R57.26 | 1 | R57.26 | DigiKey |
| DigiKey Total |  |  | R284.71 |  |
| Battery | R129 | 1 | R129 | Microrobotics |
| Battery Charger | R109 | 1 | R109 | Microrobotics |
| Potentiometer | R23 | 1 | R23 | Microrobotics |
| Switch | R16.10 | 1 | R16.10 | Microrobotics |
| Microrobotics Total |  |  | R277.10 |  |
| Total |  |  | R |  |

SOFTWARE

Complementary Filter vs Kalman Filter

- Both are sensor fusion algorithms used primarily to combine gyroscope and accelerometer data. The main difference is that a Complementary Filter is a simple, computationally light, heuristic approach, whereas a Kalman Filter is an advanced, probabilistic method that dynamically adjusts based on noise and uncertainty.

Complementary Filter

- Implementation goes here.

Kalman Filter

- Can try Complementary first and maybe improve with Kalman later.

- https://youtu.be/DQw4cNx9wdw?si=WBi8i-YrmZRtg9gD - Kalman Filter Video.

Motor Driver

- analogWrite(pin, value)

- Pin must be a PWM-capable pin (marked with ~ symbol)

- Value: an integer from 0 to 255 (0 means 0 V or 0 % duty cycle and 255 means 5 V or 100 % duty cycle)

- If it receives 7.4 V from the battery, a duty cycle of 81% will give an effective voltage of 6 V.

- I.e. a value of 207

- Measure to make sure!

Control Algorithm

- COM angle is the reference you want to set it to?

- Two nested PID loops?

Notes

- Whatever you do, keep the Gyro as close to the middle of the axle as possible and lay it flat with the pins sticking up. don't mount the Gyro halfway up or you will get an error with your readings.

QUESTIONS

- Ask Claude if my planned set up will work – Only potential issue is RPM range of current motors (200 rpm). Most successful projects use higher speed motors (e.g. 300 to 500 rpm range). Shouldn’t be impossible but will make PID tunning more difficult (Tips and tricks video uses this motor).